Main Features

# WS2814F

Single Line256Grayscale four channels

## Constant currentleddriveIC

Translated from Chinese (Simplified) to English - www.onlinedoctranslator.com

R,G,B,WOutput port withstand voltage20V,DINPort withstand voltage9V.

- The chip has a built-in voltage regulator.24VThe following power supplies only need to connect resistors toIC VDDNo external voltage regulator tube is required.

- Built-in signal shaping circuit, any pixel receives a signal and then outputs it after waveform shaping, ensuring that line waveform distortion will not accumulate.

- Built-in power-on reset and power-down reset circuits.

- PWMThe control end can achieve256Level adjustment, scanning frequency2KHz.

- The serial cascade interface can complete data reception and decoding through one signal line.

- The transmission distance between any two points does not exceed2No additional circuit is required.

- The color of light is highly consistent and cost-effective.

- When the refresh rate30Frames/second, the number of cascades is not less than1024point.

- Data transmission speed can reach800Kbps.

Main application areas

- ledFull color luminous character light string,ledFull color soft light bar hard light bar,ledGuardrail pipe.

###### ledPoint light source,ledPixel screen,ledSpecial-shaped screen.

Product Overview

###### WS2814It is a four-channelledDrive control dedicated circuit, the chip contains intelligent digital interface data latch signal shaping amplification drive circuit,

High-precision internal oscillator and20VHigh-voltage programmable constant current output driver and high-precision constant current control module effectively ensure the pixel

The color of the point light is highly consistent.

The data protocol uses a single-line return-to-zero code communication method. After the chip is powered on and reset,DINThe end receives the data transmitted from the controller and first sends

Coming32bitAfter the data is extracted by the first chip, it is sent to the data latch inside the chip, and the remaining data is shaped and amplified by the internal shaping circuit.

###### After the bigDOThe port starts forwarding the output to the next cascaded pixel point. After each pixel point is transmitted, the signal decreases.32bit. Chip uses

The automatic shaping and forwarding technology makes the cascade number of the pixel points not limited by the signal transmission, but only limited by the signal transmission speed requirement.

The data latch inside the chip is based on the received32bitData, inOUTR,OUTG,OUTB,OUTWThe control end generates different Duty cycle control signal, waitingDINTerminal InputRESETWhen a signal is received, all chips will send the received data to each segment synchronously. After the end, new data is received again.32bitAfter the data isDOThe port forwards data. The chip does not receiveRESETBefore the code,

OUTR,OUTG,OUTB,OUTWThe original output of the pin remains unchanged.280μsAbove low levelRESETAfter the code is

#### Received32bit PWMData pulse width output toOUTR,OUTG,OUTB,OUTWOn the pin.

Terminal arrangement

Lead-out terminal function

| Pin Number | symbol | Pin Name | Functional Description |
| --- | --- | --- | --- |
| 1 | OUTR | ledDriver Output | RED(red)PWMControl output |
| 2 | OUTG | ledDriver Output | GREEN(green)PWMControl output |
| 3 | OUTB | ledDriver Output | BLUE(blue)PWMControl output |
| 4 | GND | land | Signal ground and power ground |
| 5 | DOUT | Data Output | Display data cascade output |
| 6 | DIN | Data Entry | Display data input |
| 7 | VDD | Logic Power Supply | ICpowered by |
| 8 | OUTW | ledDriver Output | WHITE(white)PWMControl output |

Maximum Ratings（TA=25℃,VSS=0V）

| parameter | symbol | scope | unit |
| --- | --- | --- | --- |
| Logic supply voltage | Vdd | +3.7~+5.3 | V |
| Logic input voltage | Vi | VDD-0.7~VDD+0.7 | V |
| R,G,B,WOutput port withstand voltage | Vout | 20 | V |
| Operating temperature | Topt | - 40〜+85 | ℃ |
| Storage temperature | Tsj | -40~+150 | ℃ |
| Electrostatic immunity | ESD | ≧4 | KV |

Electrical parameters（TA=25℃,VDD=4.5~5.5V,VSS=0V）

| parameter | symbol | Minimum | typical | maximum | unit | Test conditions |
| --- | --- | --- | --- | --- | --- | --- |
| R,G,B,WLow Level output current | Iol | 15.5 | 16.5 | 17.5 | mA |  |
| Low level output current | Idout | 10 | — — | — — | mA | Vo=0.4V,DOUT |
| Input Current | Ii | — — | — — | ±1 | µA | Vi=Vdd/Vss |
| High level input | Vih | 0.7VDD | — — | — — | V | Din |
| Low level input | Vil | — — | — — | 0.3 VDD | V | Din |
| Hysteresis voltage | Vh | — — | 0.35 | — — | V | Din |

Switching Characteristics（TA=25℃,VDD=4.5~5.5V,VSS=0V）

| parameter | symbol | Minimum | typical | maximum | unit | Test conditions |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission delay time | tPLZ | — — | — — | 300 | ns | CL=15pF, DIN→DOUT, RL=10KΩ |
| Fall time | tTHZ | — — | — — | 120 | µs | CL=300pF, OUTR/OUTG/OUTB/OUTW |
| Data transfer rate | FMAX | 600 | — — | — — | Kbps | Duty Cycle50% |
| Input Capacitance | Ci | — — | — — | 15 | pF | — — |

### Data transfer time

| T0H | 0Code, High level time | 220ns~380ns |
| --- | --- | --- |
| T1H | 1Code, High level time | 580ns~1µs |
| T0L | 0Code, low level time | 580ns~1µs |
| T1L | 1Code, low level time | 580ns~1µs |
| RES | Frame unit, low level time | 280µsabove |
| Tdata | Data cycle | ≥1.25us |

##### Timing waveform

Input code type:	Connection method:

Data transmission method

Note:D1forMCUThe data sent by the end,D2,D3,D4Automatically shapes forwarded data for cascaded circuits.

32bitData Structure

| W7 | W6 | W5 | W4 | W3 | W2 | W1 | W0 | R7 | R6 | R5 | R4 | R3 | R2 | R1 | R0 | G7 | G6 | G5 | G4 | G3 | G2 | G1 | G0 | B7 | B6 | B5 | B4 | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note: High position first, according toWRGBThe data is sent in the order specified.

Typical application circuit

Package diagram and parameters

##### -FSOP8Encapsulation

| Version Number | state | Summary of the revised content | Revision Date | Revised by | Approver |
| --- | --- | --- | --- | --- | --- |
| V1.0 | N | New | 20221027 | Hu Jin | Yu Xinghui |
| V1.1 | M | Modify pin definition | 20230328 | Hu Jin | Yu Xinghui |

File Change Log

Note: Initial version numberV1.0; After each revision is approved, the version number is sequentially increased by "0.1”;

The statuses include:N--New,A--Increase,M--Revise,D--delete.